微算機系統 2025

MICROPROCESSOR SYSTEMS

4-stage Pipeline CPU

Group 34

110590042 柳艾德

112590002 林羿安

Content

| Instruction | Instruction code |  |  | usage | Result |
| --- | --- | --- | --- | --- | --- |
|  | opcode | Rs | Rt |  |  |
| Load | 0000 | 2bit | 2bit | Load Rs Data | Rs←Data |
| Move | 0001 |  |  | Move Rs, Rt | Rs←Rt |
| Add | 0010 |  |  | Add Rs, Rt | Rs←Rs + Rt |
| And | 0011 |  |  | And Rs, Rt | Rs←Rs & Rt |
| Sub(A-B) | 0101 |  |  | Sub Rs, Rt | Rs←Rs – Rt |
| Sub(B-A) | 1001 |  |  | Sub Rs, Rt | Rs←Rt - Rs |
| Nor | 0110 |  |  | Nor Rs, Rt | Rs←Rs Nor Rt |
| Slt | 0100 |  |  | Slt Rs, Rt | if (Rs<Rt) Rs=1;else Rs=0 |
| Div | 1000 |  |  | Div Rs, Rt | Rs←Rs / Rt |

This week’s lab focuses on designing a simple 4-stage pipelined CPU capable of executing basic arithmetic and logic operations. The CPU includes a small register file with four 8-bit registers and uses RS and RT fields to select source and destination operands. Instructions are input manually through switches and move through stages for fetching, decoding, execution, and write-back. Forwarding logic is implemented to handle data hazards without stalling, and LEDs are used to show the status of each pipeline stage, while seven-segment displays reflect register values and input data in real time.

Process & Results

處理流程利用 process 同時執行的特性來模擬 pipeline 的結構，並將 each stage 所需的 register content 用 signal 的方式儲存，方便資料的傳遞。並利用 valid buffer 來控制 stage 的啟動和關閉。

- Each Stage

| IF： | 依照 Lab8 Port Map 讀取指令 |
| --- | --- |
| ID： | 把 Fetch 的 Instruction 所得的 Rs & Rt Address 轉換成 Data |
| EXE： | 將 Data(Rs & Rt) 通過 Opcode 取得 Result |
| WB： | 將 Result 寫回 Rs 所指向的 Register File (燈亮即寫回) |

- Display

- PS

- Opcode “1111” 為  NOP

- Div 未實作

- Hazard 未實作

- Test Data

[TEST 1]

Clk = 1, Load 00, 0x89

Clk = 4, Move 01, 00

Clk = 7

[TEST 2]

Clk = 1, Load 00, 0x08

Clk = 4, Add 00, 01

Clk = 7, Result = 0x08 + 0x89 = 0x91

Code

LIBRARY IEEE; USE IEEE.STD_LOGIC_1164.ALL; USE IEEE.NUMERIC_STD.ALL; USE work.MyPackage.ALL; ENTITY SampleCPU IS PORT ( -- button input clk : IN STD_LOGIC; -- switches input Opcode : IN STD_LOGIC_VECTOR(3 DOWNTO 0); Rs : IN STD_LOGIC_VECTOR(1 DOWNTO 0); Rt : IN STD_LOGIC_VECTOR(1 DOWNTO 0); Data : IN STD_LOGIC_VECTOR(7 DOWNTO 0); -- Hex output HEX0, HEX1, HEX2, HEX3, HEX4, HEX5 : OUT STD_LOGIC_VECTOR(6 DOWNTO 0); -- Stage LEDG : out STD_LOGIC_VECTOR(3 DOWNTO 0) -- LEDR : out std_logic; ); END SampleCPU; ARCHITECTURE Behavioral OF SampleCPU IS type reg_array is array (0 to 3) of STD_LOGIC_VECTOR(7 downto 0); signal reg_file : reg_array := (others => (others => '0')); -- r0 ~ r3 signal valid : std_logic_vector(3 downto 0) := (others => '0'); -- IF/ID pipeline registers signal IF_ID_Data : STD_LOGIC_VECTOR(7 downto 0); signal IF_ID_Opcode : STD_LOGIC_VECTOR(3 downto 0); signal IF_ID_Rs : STD_LOGIC_VECTOR(1 downto 0); signal IF_ID_Rt : STD_LOGIC_VECTOR(1 downto 0); -- ID/EXE pipeline registers signal ID_EXE_Opcode : STD_LOGIC_VECTOR(3 downto 0); signal ID_EXE_Rs : STD_LOGIC_VECTOR(1 downto 0); signal ID_EXE_Rt : STD_LOGIC_VECTOR(1 downto 0); signal ID_EXE_Rs_Data : STD_LOGIC_VECTOR(7 downto 0); signal ID_EXE_Rt_Data : STD_LOGIC_VECTOR(7 downto 0); -- EXE/WB pipeline registers signal EXE_WB_Rs : STD_LOGIC_VECTOR(1 downto 0); signal EXE_WB_Result : STD_LOGIC_VECTOR(7 downto 0); signal rs_seg, rt_seg : STD_LOGIC_VECTOR(7 downto 0); BEGIN -- IF process(clk) begin if rising_edge(clk) then if Opcode = "1111" then -- NOP valid(0) <= '0'; else valid(0) <= '1'; IF_ID_Data <= Data; IF_ID_Opcode <= Opcode; IF_ID_Rs <= Rs; IF_ID_Rt <= Rt; end if; end if; end process; -- ID process(clk) begin if rising_edge(clk) then if valid(0) = '0' then valid(1) <= '0'; else valid(1) <= '1'; ID_EXE_Opcode <= IF_ID_Opcode; ID_EXE_Rs <= IF_ID_Rs; ID_EXE_Rt <= IF_ID_Rt; if IF_ID_Opcode = "0000" then -- Load ID_EXE_Rs_Data <= IF_ID_Data; else ID_EXE_Rs_Data <= reg_file(to_integer(unsigned(IF_ID_Rs))); end if; ID_EXE_Rt_Data <= reg_file(to_integer(unsigned(IF_ID_Rt))); end if; end if; end process; -- EXE process(clk) variable result : std_logic_vector(7 downto 0) := (others => '0'); begin if rising_edge(clk) then if valid(1) = '0' then valid(2) <= '0'; else valid(2) <= '1'; case ID_EXE_Opcode is when "0000" => -- Load rs <= data result := ID_EXE_Rs_Data; when "0001" => -- Move rs <= rt result := ID_EXE_Rt_Data; when "0010" => -- Add result := std_logic_vector(unsigned(ID_EXE_Rs_Data) + unsigned(ID_EXE_Rt_Data)); when "0011" => -- And result := std_logic_vector(unsigned(ID_EXE_Rs_Data) and unsigned(ID_EXE_Rt_Data)); when "0101" => -- Sub: rs - rt result := std_logic_vector(unsigned(ID_EXE_Rs_Data) - unsigned(ID_EXE_Rt_Data)); when "1001" => -- Sub: rt - rs result := std_logic_vector(unsigned(ID_EXE_Rt_Data) - unsigned(ID_EXE_Rs_Data)); when "0110" => -- Nor result := std_logic_vector(not (unsigned(ID_EXE_Rs_Data) or unsigned(ID_EXE_Rt_Data))); when "0100" => -- Slt if unsigned(ID_EXE_Rs_Data) < unsigned(ID_EXE_Rt_Data) then result := "00000001"; else result := "00000000"; end if; when "1000" => -- Div -- TODO: when others => result := (others => '0'); end case; EXE_WB_Rs <= ID_EXE_Rs; EXE_WB_Result <= result; end if; end if; end process; -- WB process(clk) begin if rising_edge(clk) then if valid(2) = '0' then valid(3) <= '0'; else valid(3) <= '1'; reg_file(to_integer(unsigned(EXE_WB_Rs))) <= EXE_WB_Result; end if; end if; end process; -- Output LEDG <= valid; -- Show the stage of the CPU rs_seg <= reg_file(to_integer(unsigned(Rs))); rt_seg <= reg_file(to_integer(unsigned(Rt))); BusHex0: HexDisplay port map(Hex => Data(3 downto 0), Seg => HEX0); BusHex1: HexDisplay port map(Hex => Data(7 downto 4), Seg => HEX1); RsHex0: HexDisplay port map(Hex => rs_seg(3 downto 0), Seg => HEX2); RsHex1: HexDisplay port map(Hex => rs_seg(7 downto 4), Seg => HEX3); RtHex0: HexDisplay port map(Hex => rt_seg(3 downto 0), Seg => HEX4); RtHex1: HexDisplay port map(Hex => rt_seg(7 downto 4), Seg => HEX5); END Behavioral;